Community Based Traditional Irrigation Schemes Performance:

A Case Study of Upper Awash River Basin of Addis Ababa Sub-Catchments

Tadesse Bekele, Ministry of Ariculture (MoA), Girma Taddese, ILRI, g.taddesse@cgiar.org  & Don Peden, d.peden@cgiar.org

Summary

The study was carried out in Addis Ababa sub-catchments along the Akaki River. The Akaki River is a tributary of Awash River in Upper Awash basin has two main catchments (Little Akaki and Big Akaki River) which drain into Aba Samuel Artificial reservoir. The traditional irrigation schemes grow vegetables along the Akaki River and supply about 30 % of the vegetable demand for Addis Ababa city. The water management in the Schemes was very poor and some of the traditional irrigations schemes are located in the river course and face frequent over flooding during the heavy rainy season. Apart from that there is great concern using fresh vegetables from this schemes as they are irrigated with municipal wastewater. Some factories dump waste product is and water directly to the river with out treating. The concentration of heavy metals and coliforms are increasing below Aba Samuel dam an out let of Akaki River. The overall information showed that the farmers get adequate benefit from traditional irrigation and hold dairy cows.  Finally the Addis Ababa sub catchments community irrigation activity should be thoroughly studied to for the benefit of the Addis Ababa community.

###### Introduction

The Awash River basin is divided into Upper, Middle and Lower Awash Basin.  The source of Awash River is the central plateau in the west of Addis Ababa and flows into Lake Abbe near Djibouti (Zewdie Abate 1994). Awash Rivers starts at an elevation of about 3,000meter in the central Ethiopian high lands, West of Addis Ababa and flows north east wards along the rift valley in to the Afar triangle where it terminate in lake Abe at an elevation of 250 meter above sea level. The main river length is about 1,200 kilometers (EVDSA 1989).

The Awash basin covers a total area of 110,000 km2and comprises three regions Oromia, Afar, Amhara, Addis Ababa city Administration and Dire Dawa council. Awash basin is intensively utilized river basin in Ethiopia due to its strategic location, access roads, available land and water resources.  Currently 21865 ha traditional and 4932ha modern small-scale irrigation schemes have been developed. Irrigation potential in Awash basin is estimated to be 206, 000 ha (Table 1).  However, the basin suffers from severe environmental degradation, annual flooding improper utilization of land and water resources, socio-economic constraints, poor agricultural practices, low yielding and community health problems. Thus this paper assesses the performance of existing community based traditional irrigation schemes in Upper Awash basin.  Finally the objectives of this work is to study the water use efficiency of traditional small irrigation schemes in Addis Ababa catchments and to study the economic benefit of traditional small scale irrigation schemes.

The study area

The study was carried in Addis Ababa sub-catchments along the Akaki River. The Akaki River is found in Upper Awash basin has two main catchments (Little Akaki and Big Akaki River) which drains into Aba Samuel Artificial reservoir. In the sub-basins of Awash Addis Ababa catchments there are more than nine perennial river tributaries, which drain to main Akaki River. The catchment’s area is estimated to be 137, 000 ha.  The studied schemes varied in size from 4ha to 150ha.  Four of the schemes have water pumps with a capacity of 30 l/sec.  The farmers are organized in traditional way of water use associations (WUAS). Each irrigation scheme has water committee.  The water committee has the major task of responsibility for water administration and management, coordinating, operation and maintenance of canals and resolution of disputes among beneficiaries.

The memberships include both men and women and consist of 1574 family heads and 7630 families. Out of these 28 % are women farmers. The farmers manage operation and maintenance works of their schemes.

Along the right and the left side of the River Akaki River 389.5 ha of land are developed under traditional small-scale irrigation system (Table 1). The traditional irrigation schemes grow vegetables along the Akaki River.

The major soil types of the study area are Pellic Vertisols and Nitosols. Pellic Vertisols are found at Akaki, just south of Addis Ababa with too shallow soil depth at 3% slope with surface cracking features.  Nitosols are found in the high land of Addis Northwest and Northeast of the catchments.  Nitosols have very good potential for agriculture and   have high water storage capacity.

The catchments has very good favorable climate for certain vegetable crops. The traditional irrigation water delivery system, irrigation frequency and water management systems are very poor.  More often the farmers spend their time in maintaining canals, which they have to build every season after the small and big rainy season.

Major crops grown

The major vegetable crop grown in the schemes are lettuce, swiss charade, carrot, kale, cabbage, potato, zikuni (cucumber), cauliflower, beans, tomato, pepper and onion. About 85% improved vegetable seeds are supplied by Agricultural Input Services Corporations (AISCO) the remaining 15% seed bulk is grown and supplied by farmers themselves (Table 2).

Crop water need

The Pen Man Montith method was used to calculate reference evapotranspiration (ETo) and Effective rainfall (Re) was calculated using USDA Soil Conservation Service method.  Crop water requirement (ETc) is calculated by the formula (ETc =ETo x Kc) and field efficiency is taken as 60 %.  Net irrigation (In) was also calculated as In = ETc-Re and approximate net irrigation depth per irrigation application was taken by assuming the root depth of each vegetable crop and soil type of the study area. Number of irrigation application was calculated from: irrigation water need over growing season / net depth of each irrigation application.

Economic Calculations:

Total cost = Labor cost (birr/ha) + Seed Cost (Birr/ha)

Net production (qt/ha) = Productivity (qt/ha) – 15% production losses

Total income (Birr/ha) = Net production (qt/ha) x Market price (birr/kg)

Net income (Birr/ha) = Total income (Birr/ha) – Total cost (Birr/ha)

Feasibility Ratio = Net income (Birr/ ha)/ total cost (Birr/ha)

Results

In general net irrigation requirement for cabbage and kale for the first season was 396 mm and the lowest irrigation water requirement was recorded for leek in the second season. The field irrigation requirement was also 673.5 mm for cabbage and kale. Similar trends were also reflected for number of irrigation application for both crops. The least number of irrigation applications was for leek in the first cropping season. Moreover the highest irrigation interval was recorded for tomato in the second cropping season and the lowest irrigation interval was observed for lettuce (Table 4). Generally Lettuce was the most important vegetable produced in all of the schemes.  The lettuce area coverage was 54.92 hectares.  The next important vegetable crop is Swiss charade 48.63 ha, carrot 45.9 ha followed by Kale, cabbage, potato, cucumber, cauliflower (Table 5).  The result showed that the crop water requirement of each crop varies.

Only labor and seed cost expenses were incurred in the calculation of   irrigation performance of the irrigation schemes. Producing cucumber appears to be more profitable for the beneficiaries with net income of 65660 birr/ha as compared to the rest of the vegetable crops (Table 6).

The cropping intensities were 200 %, but for Swiss charade and cucumber was 100%.  The calculated net income was 694.4 birr/month and the total income was 1, 3114, 512.26 birr/year for the whole schemes. The individual farmer may get 694.4 birr/month, which is considerable high income for ordinary Ethiopian farmer (Table 7).  Some indicators were also calculated to determine the water use performance of the crops in the schemes (Table 8).

Discussion

Ethiopia is endowed with rich water resources, but yet untapped natural resources that could have largely contributed to the over all national economic development on a sustainable basis. Ethiopian agriculture mainly depends on rain fed, which is erratic.   As a result repetitive drought, shortage of food and malnutrition became the main features of the country.   Current agricultural strategy and policy of the government promotes irrigated agriculture in all potential river basins. The modern small irrigation schemes and traditional irrigation are sustainable and are easily manageable by the farmers.

The key factor for the development of small-scale irrigation around Addis Ababa city is the large demand for fresh vegetables for the local market.  However, the vegetables grown under traditional irrigation schemes and the yield obtained are not high due to poor agronomic practices, lack of technological inputs, and low level of water use efficiency.

Most of the scheme sites are found along the river mouth and exposed to flood hazard.  During the rainy season the fields are water logged and are not utilized for crop production. Traditional irrigation also faces common problems such as paucity in land holding, insufficient capital, inequitable water use and social conflicts in utilization of water.

Irrigation water scheduling also is not well practiced and there is a possibility of applying of excess water to crops which compatible with the crop water requirement.   At the beginning of the growing season, the amount of water given per irrigation application, or the irrigation depth is small and is given frequently.  Farmers who live around Addis Ababa are within the orbit of the existing market information system get reliable sources of income.

The development of irrigated agriculture around Addis Ababa city needs a systematic cost benefit analysis and the realization of profitability for market oriented production and the attainment of sustainable development.   The market oriented irrigated agricultural activity in general should consider proper estimation of labor, material, inputs, transport, maintenance, store, operation and investment costs in their cost benefit analysis to optimize profitability. Give priority to the demand of the market in planning and producing vegetable products.

Farmers in irrigated schemes got better market access much income than that of rain fed farming (Table 6).  This gives every reason for the policy makers to channel more concern to smallholder irrigation development.

Despite its success and growing importance of per-urban and urban small traditional irrigation it is still subject to numerous constraints.  Among them are in sufficient access to non-contaminated or clean water, uncertainty about land tenure, and poor water use efficiency. The majority of beneficiaries hold less than 0.5ha per household and land fragmentation has been the basic problem for irrigation development. Most of the farmland is found along the riverbed where irregular flood hazard can easily wipe out the farmland and high losses of water. Main feeder cannels are often poorly constructed and maintained and have poor water distribution system within the potential command area. This takes considerable amount of time, efforts and money to make them productive. The schemes lack proper maintenance and reconstruction of the headwork and are subject to uncontrolled use of water for irrigation.   There is high loss of irrigation water in the traditional irrigation schemes when crossing gullies.  Frequently inequitable distribution of water between upstream users and down stream users causes conflicts.

Women perform important agricultural activities such as seeding, weeding, harvesting, threshing, mulling and other related works.  In areas under study women's participation in the irrigation schemes is encouraging.  Equal farm level access to water and related obligation equal participation in "forums" or network for collective water management equality at leadership level gender composition of leader in the WUA's. Farmers in conjunction with vegetable growing they have also livestock of various use. Women play greater role in holding dairy cows than men in the schemes.  The milk is supplied to the town and milk-processing factory in Addis Ababa.

Measures to boost agricultural production play a very important role in achieving development policy goals by reducing food in security, and increasing incomes and employment opportunities.

Expansion of new industries and disposal of industrial wastes to the Awash River basin is great concern to the nation (Girma Taddese 2001).  Food and beverage factories tend to discharge heavy organic pollutions and dyes from textile factories.  All these disposals are damped to Akaki River, which is the main tributary of Awash River.  This river is the main source of irrigation water in the studied traditional community based irrigation schemes, where 30% from vegetable supplier to the city. In this regard, the issue of health is critical in urban and per-urban agriculture where water is polluted by industrial wastes, application in farming of such untreated waste can pose significant health risks both to those who have direct contact with it and also to the general public who are affected through the food chain link (Furedy et al. 1999).

Small-scale irrigation schemes in the sub-basin leave much to be designed in terms of efficient water use and prevention of harmful conditions resulting from water logging and pollutant of heavy metals.  It will be necessary to introduce better irrigation methods and water management techniques.  Diversion of perennial river, harvesting and storage of water from frequent flash floods, and the utilization of ground water for irrigation should be targeted for future agricultural development and food security. In Ethiopia, as in the rest of Africa, many irrigation projects have proved to be unsustainable. Co-operation was unpopular with peasants and many were reluctant to invest their labour and energy in the irrigation venture.

All too often, state agents harbour strong paternalist attitudes towards peasants. There is frequently a tendency on their part to define the rules and set the guidelines governing irrigation schemes without involving the peasants.

Water security, like land tenure security, requires that users must have reliable and timely access to water adequate for their needs.

Reference

Alemtsehaye Biru. 2002. Assessment of the fertility and pollution status of irrigated vegetable framers around Addis Ababa. Addis  Ababa City Government Agricultural Office.

Furedy C, V Maclaren and  J Whitney. 1999. Reuse of waste for food Production in Asian Cities: Health and economic Perspectives. In: Koc M, R Macrae, LJA Mougeot and J Welsh (eds), For Hunger-proof Cities. Ottawa: International Development Research Centre, pp 136-144.

Girma Taddese. 2001. Land degradation: A challenge to Ethiopia. Environmental Management 27: 815-824.

Perrier. E. R. and Salkin. A. B. 1987. Supplementary irrigation in the Near East and North Africa. Kluwer Academic Publishers.

Zewdie Abate. 1994. Water resources Development in Ethiopia. An evaluation of present experience and future planning concepts.

Table 1. Small scale-irrigation development in Awash Basin.

| Regions within the basins | Small scale irrigation |  |  |  | Total irrigated |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Traditional |  | Modern |  |  |  |
|  | Area (ha) | Beneficiaries | Area (ha) | Beneficiaries | Area (ha) | Beneficiaries |
| Oromia | Area (ha) | 24,282 | 1,249 | 2409 | 7471 | 26,691 |
| Afar | 3114 | 12,581 | - | - | 3114 | 12,581 |
| Amhara | 11,273 | 60,114 | 3337 | 11,608 | 14,610 | 71,722 |
| Addis Ababa | 389.6 | 977 | - | - | 389.6 | 977 |
| Dire Dawa | 866 | 4342 | 345.5 | 1629 | 1211.5 | 5971 |
| Grand total | 21864.6 | 102,296 | 4931.5 | 15,646 | 26796.1 | 117,942 |

Table 2.  Traditional irrigation schemes in the sub-basin of Addis Ababa catchments

| Name of scheme | River | Districts | Irrigated Area (ha) | Beneficiaries |  |  | Type of Management | Irrigation system |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Women | Men | Total |  |  |
| Shankla Ena Kacha Fabrica | Shankla | Coca Cola Fabrica | 8.5 | 11 | 41 | 52 | Farmers | Surface irrigation |
| Tinshu Akaki Ena Keranio | T. Akaki | Keranyo | 7.5 | 3 | 23 | 26 | Farmers | Surface irrigation |
| Tinshu Akaki Mekanisa Goffa & Furi Saris | T. Akaki | Goffa Sefer | 150 | 40 | 204 | 244 | Farmers | Surface irrigation |
| Kebena Ena Bulubla | Kebena & Bulbula | Peackock Park | 7.5 | 6 | 25 | 31 | Farmers | Pump operated |
| Kolfae Ena Lideta | T. Akaki | Kolfae | 51 | 17 | 92 | 109 | Farmers | Surface irrigation |
| Fanta | Akaki | Akaki | 10 | - | 28 | 28 | Farmers | Surface irrigation |
| Mekana Brhan Tesfa | Akaki | Akaki | 12 | 45 | 20 | 65 | Farmers | Pump operated |
| Mekana Yersha Limat Abat Turptengoch | Akaki | Akaki | 5 | 5 | 17 | 22 | Farmers | Surface irrigation |
| Fnote Selam | Akaki | Akaki | 5 | 5 | 60 | 65 | Farmers | Surface irrigation |
| Abat Enat Turetngoch | Akaki | Akaki | 5.5 | 99 | 126 | 225 | Farmers | Pump operated |
| Chello | Akaki | Akaki | 4 | 90 | - | 90 | Farmers |  |
| Bole Lemi | Akaki | Bole | 4 | 20 | - | 20 | Farmers | Pump operated |
|  | Akaki |  |  |  |  |  |  |  |
| Other small holders |  |  | 119.5 | 100 | 497 | 597 | Farmers | Surface irrigation |
| Total |  |  | 389.5 | 441 | 1133 | 1574 |  |  |

Table 3. Improved vegetable seed recommended for irrigation schemes for the study area.

| Vegetable | Improved variety | Seeding rate (kg/ha) | Price (birr/kg) | Total price (birr/ha) |
| --- | --- | --- | --- | --- |
| Potato | Tollcha | 1500-2000 | 2.75 | 4125 |
| Tomato | Marglov, money maker, Roma.VF | 0.22 | 490 | 107.8 |
| Carrot | Nantes | 3.6 | 176 | 792 |
| Onion | Red croyale | 3.5 | 260 | 910 |
| Pepper | Marecofana | 0.6 | 30 | 18 |
| Beet root | Detroit dark red | 16 | 150 | 2400 |
| Cabbage | Copenhagen | 0.4 | 400 | 160 |
| Lettuce | Great Leakes mesa | 1.7 | 180 | 306 |
| Leek |  | 3.5 | 200 | 700 |
| S. Charade | Ford hook giant | 15 | 185 | 2775 |
| Kale | Zkuala or local Guraghe | 0.4 | 40 | 16 |
| Cauliflower | Snow ball or Glacier | 0.4 | 350 | 140 |

Source: Addis Ababa City Agricultural Office (2002).

Table 4. Crop water Requirement for Vegetable growers in Addis Ababa sub-catchments of Upper Awash Basin.

| Crop | Season | ETo | ETc | Rainfalla | Rainfalle | Netirreq | Fieldirreq. | Aappirrno | Irrinter |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (mm) | (mm) | (mm) | (mm) | (mm) | (mm) |  |  |
| Potato | 1 | 515 | 422.3 | 68 | 65.7 | 356.6 | 606.2 | 20 | 6 |
|  | 2 | 542 | 450.8 | 274 | 241.5 | 209.3 | 355.8 | 12 | 10 |
| Tomato | 1 | 515 | 397.1 | 68 | 65.7 | 331.4 | 563.4 | 11 | 11 |
|  | 2 | 542 | 423.8 | 274 | 241.5 | 182.3 | 309.9 | 6 | 20 |
| Carrot | 1 | 515 | 348.5 | 68 | 65.7 | 300.3 | 510.5 | 10 | 9 |
|  | 2 | 542 | 370.4 | 274 | 241.5 | 203.8 | 346.4 | 7 | 13 |
| Onion | 1 | 515 | 490.7 | 68 | 65.7 | 344 | 384.8 | 19 | 6 |
|  | 2 | 542 | 438.4 | 274 | 241.5 | 196.9 | 334.7 | 11 | 11 |
| Pepper | 1 | 515 | 408.7 | 68 | 65.7 | 343 | 583.1 | 12 | 6 |
|  | 2 | 542 | 438.3 | 274 | 241.5 | 196.8 | 334.6 | 7 | 11 |
| Beets | 1 | 515 | 321.5 | 68 | 65.7 | 273.3 | 464.6 | 9 | 6 |
|  | 2 | 542 | 345.6 | 274 | 241.5 | 179 | 304.2 | 6 | 11 |
| Cabbage | 1 | 515 | 461.9 | 68 | 65.7 | 396.2 | 673.5 | 22 | 8 |
|  | 2 | 542 | 429.1 | 274 | 241.5 | 187.6 | 318.8 | 11 | 14 |
| Lettuce | 1 | 515 | 342.2 | 68 | 65.7 | 294 | 499.7 | 17 | 5 |
|  | 2 | 542 | 363 | 274 | 241.5 | 196.4 | 333.9 | 11 | 8 |
| Cucumber | 1 | 515 | 351.7 | 68 | 65.7 | 286 | 486.2 | 10 | 6 |
|  | 2 | 542 | 375.1 | 274 | 241.5 | 133.6 | 277 | 5 | 11 |
| Leek | 1 | 515 | 293.9 | 68 | 65.7 | 228.2 | 387.9 | 13 | 6 |
|  | 2 | 542 | 288.2 | 274 | 241.5 | 67.6 | 114.9 | 4 | 11 |
| Swiss Charde | 1 | 528 | 260.4 | 171 | 153.6 | 209.7 | 356.5 | 12 | 9 |
| Kale | 1 | 515 | 461.9 | 68 | 65.7 | 396.2 | 673.5 | 22 | 8 |
|  | 2 | 542 | 429.1 | 274 | 241.5 | 187.6 | 317.8 | 11 | 14 |
| Pumpkin | 1 | 529 | 244.2 | 171 | 153.6 | 193.5 | 329 | 11 | 16 |
| Cauliflower | 1 | 515 | 449.3 | 68 | 65.7 | 383.6 | 652.1 | 22 | 6 |
|  | 2 | 542 | 464.9 | 274 | 241.5 | 223.4 | 379.7 | 13 | 11 |
| Phaseolus | 1 | 515 | 390.8 | 68 | 65.7 | 325.1 | 552.6 | 18 | 6 |
|  | 2 | 542 | 433.3 | 274 | 241.5 | 191.8 | 324 | 11 | 11 |

Table 5.  Mean vegetable productivity under traditional irrigation schemes

| Vegetables | Cultivated Area (ha) | Cropping pattern (%) | Total production (qt) | Productivity (qt/ha) |
| --- | --- | --- | --- | --- |
| Potato | 48.7 | 12.5 | 9740 | 200 |
| Tomato | 15.03 | 3.85 | 2254.5 | 150 |
| Carrot | 45.9 | 11.8 | 9180 | 200 |
| Onion | 7 | 1.8 | 616 | 88 |
| Pepper | 5.75 | 1.48 | 172.5 | 30 |
| Beet root | 25.3 | 6.49 | 7590 | 300 |
| Cabbage | 32.57 | 8.36 | 9771 | 300 |
| Lettuce | 54.92 | 14.1 | 3295.2 | 60 |
| Leek | 16 | 4.11 | 1600 | 100 |
| Swiss Charde | 48.63 | 12.48 | 7294.5 | 150 |
| Pumpkin | 3.6 | 0.92 | 1440 | 400 |
| Cucumber (Zikuni) | 10.31 | 2.65 | 4124 | 400 |
| Phaseolus | 10.27 | 2.64 | 215.67 | 21 |
| Kale | 39.06 | 10.03 | 2343.6 | 60 |
| Cauliflower | 26.56 | 6.79 | 1567.04 | 59 |
| Total | 389.6 | 100 | 61204.01 | 2518 |

Source: Bureau of Agriculture Addis Ababa, 2003

Tabl 6.  Labor and seed cost, value of crops and net incomes.

| Vegetables | Labour cost (Birr/ha) | Seed cost (birr/ha) | Total cost (Birr/ha) | Productivity (qt/ha) | Losses (%) | Net production (qt/ha) | Market price (birr/ kg) | Total income (birr/ha) | Net income (birr/ha) | Feasibility ratio |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Lettuce | 915 | 306 | 1221 | 150 | 15 | 127.5 | 2 | 25,500 | 24,279 | 19.9 |
| Swiss Charade | 915 | 2775 | 3690 | 150 | 15 | 127.5 | 2 | 25,500 | 21,810 | 5.9 |
| Carrot | 1090 | 1056 | 2146 | 200 | 15 | 170 | 1 | 17,000 | 14,854 | 6.9 |
| Cabbage | 915 | 160 | 1075 | 300 | 15 | 255 | 1 | 25,500 | 24,425 | 22.7 |
| Kale | 915 | 16 | 931 | 60 | 15 | 51 | 2 | 10,200 | 9,269 | 10.0 |
| Potato | 960 | 4125 | 5085 | 200 | 15 | 170 | 1 | 17,000 | 11,915 | 2.3 |
| Beet Root | 1090 | 2400 | 3490 | 300 | 15 | 255 | 1 | 25,500 | 22,010 | 6.3 |
| Tomato | 1095 | 108 | 1203 | 150 | 15 | 127.5 | 1.25 | 15,938 | 14,735 | 12.2 |
| Onion | 1090 | 910 | 2000 | 88 | 15 | 74.8 | 2 | 14,960 | 12,960 | 6.5 |
| Pepper | 915 | 18 | 933 | 30 | 15 | 25.5 | 2.5 | 6,375 | 5,442 | 5.8 |
| Leek | 915 | 700 | 1615 | 100 | 15 | 81 | 2 | 16,200 | 14,585 | 9.0 |
| Pumpkin | 1200 | 1250 | 2450 | 400 | 15 | 340 | 1 | 34,000 | 31,550 | 12.9 |
| Cucumber | 1090 | 1250 | 2340 | 400 | 15 | 340 | 2 | 68,000 | 65,660 | 28.1 |
| Phaslalus | 1090 | 180 | 1270 | 21 | 15 | 17.85 | 2.5 | 4,463 | 3,193 | 2.5 |
| Cauliflower | 1090 | 140 | 1230 | 60 | 15 | 51 | 3 | 15,300 | 14,070 | 11.4 |
| Total | 15285 | 15394 | 30679 | 2609 | 225 | 2213.65 | 26.25 | 321,435 | 290756 |  |

Table 7.  Mean net income from the irrigation schemes.

| Vegetables | Plot size (ha) | Net income (birr/ha) | Total net income (birr) | Cropping intensity % | Total net income (birr/ year) | Average income per farmer (birr/month) |
| --- | --- | --- | --- | --- | --- | --- |
| Lettuce | 54.92 | 24,279 | 1333402.7 | ± 200 | 2666805.36 | 141.2 |
| S. Charade | 48.63 | 21,810 | 1060620.3 | ± 100 | 1060620.3 | 56.2 |
| Carrot | 45.9 | 14,854 | 681798.6 | ± 200 | 1363597.2 | 72.2 |
| Cabbage | 32.57 | 24,425 | 795522.3 | ± 200 | 1591044.5 | 84.2 |
| Kale | 39.06 | 9,269 | 362047.1 | ± 200 | 724094.28 | 38.3 |
| Potato | 48.7 | 11,915 | 580260.5 | ± 200 | 1160521 | 61.4 |
| Beet Root | 25.3 | 22,010 | 556853.0 | ± 200 | 1113706 | 59.0 |
| Tomato | 15.03 | 14,735 | 221459.5 | ± 200 | 442919.07 | 23.5 |
| Onion | 7 | 12,960 | 90720.0 | ± 200 | 181440 | 9.6 |
| Pepper | 5.75 | 5,442 | 31291.5 | ± 200 | 62583 | 3.3 |
| Leek | 16 | 14,585 | 233360.0 | ± 200 | 466720 | 24.7 |
| Pumpkin | 3.6 | 31,550 | 113580.0 | ± 100 | 113580 | 6.0 |
| Cucumber (Zukuni) | 10.31 | 65,660 | 676954.6 | ± 200 | 1353909.2 | 71.7 |
| Pha Seolus | 10.27 | 3,193 | 32787.0 | ± 200 | 65573.95 | 3.5 |
| Cauliflower | 26.56 | 14,070 | 373699.2 | ± 200 | 747398.4 | 39.6 |
| Total | 389.6 | 290756 | 7144356.3 |  | 13114512.26 | 694.4 |

Table 8. Indicators for comparing performance of community based traditional irrigated of Addis Ababa sub-catchments in Upper Awash Basin

|  | Output | Output | Output | Relative |
| --- | --- | --- | --- | --- |
|  | (Birr/ha) | per unit | per unit | Water |
|  |  | water | irrigation | supply |
|  |  | consumed | supply |  |
|  |  | (Birr/m3 ) | (Birr/m3 ) |  |
| Beet Root | 44020 | 246.23 | 144.86 | 7.24 |
| Cabbage | 48850 | 272.53 | 160.34 | 6.90 |
| Carrot | 29708 | 270.50 | 159.13 | 7.45 |
| Cauliflower | 28140 | 123.13 | 72.44 | 6.98 |
| Cucumber | 131320 | 322.67 | 177.40 | 6.20 |
| Kale | 18538 | 124.03 | 73.04 | 6.90 |
| Leek | 29170 | 157.78 | 92.82 | 5.61 |
| Lettuce | 48558 | 543.80 | 319.80 | 7.39 |
| Onion | 25920 | 33.54 | 25.22 | 6.15 |
| Pepper | 10884 | 11.59 | 6.82 | 6.74 |
| Phaslalus | 6385 | 12.69 | 7.48 | 6.65 |
| Potato | 23830 | 205.08 | 120.64 | 6.83 |
| Pumpkin | 31550 | 58.70 | 34.52 | 8.55 |
| Swiss Charade | 21810 | 505.78 | 297.51 | 8.64 |
| Tomato | 29469 | 86.22 | 50.72 | 6.71 |
| Total | 528152 | 2974.275558 | 1742.740726 | 104.928489 |